⚡

SHEMS CONTROL HUB

Smart Home Energy Management System

Setup & User Guide

JKUAT Final Year Project NLP-Enhanced Bilingual Relay Controller mBERT · Whisper STT · Arduino Uno · 100% Offline

# 1. System Overview

SHEMS is a bilingual (English + Swahili) voice-controlled smart home system built as a JKUAT final year project. It lets you control a physical DC light bulb connected through an Arduino Uno via natural language commands — spoken or typed — while logging every interaction and tracking energy consumption in real time.

What the system does, end to end:

- Captures your voice through the browser microphone or terminal

- Transcribes it offline using OpenAI Whisper (no internet required)

- Classifies the intent using a fine-tuned mBERT model trained on English and Swahili smart home commands

- Sends a serial command to the Arduino Uno, which toggles the relay and controls the physical bulb

- Logs every transaction to a local SQLite database and displays live telemetry on a web dashboard

Four supported intents:

| Intent | Example Commands |
| --- | --- |
| TURN_ON | Turn the light on — e.g. "Turn on the light", "Washa taa" |
| TURN_OFF | Turn the light off — e.g. "Switch off the bulb", "Zima taa" |
| GET_STATUS | Ask current state — e.g. "Is the light on?", "Hali ya taa ni nini?" |
| GET_ADVICE | Energy saving tips — e.g. "How can I save energy?", "Nipe ushauri wa nishati" |

# 2. System Architecture

The system consists of three layers that work together:

📡 NLP Layer (runs on your PC / laptop)

Flask API server (app.py) — receives commands via HTTP OpenAI Whisper — transcribes voice to text, fully offline mBERT classifier — predicts intent from bilingual text SQLite database — logs all transactions and energy sessions Web dashboard — real-time telemetry at http://127.0.0.1:5000

🔌 Hardware Layer (Arduino Uno)

Receives serial commands from Flask over USB: RELAY:1:ON / RELAY:1:OFF Toggles the relay module connected to digital pin 7 Responds with ACK:ON or ACK:OFF to confirm actuation Relay switches the DC bulb circuit

🌍 Interface Layer (browser / terminal)

Web dashboard at http://127.0.0.1:5000 — voice recording, text commands, live logs voice_client.py — standalone terminal STT client test_api.py — automated command test battery

## 2.1  Data Flow

Every command follows this exact path through the system:

DATA FLOW

Browser mic / voice_client.py

│

▼

Whisper STT  →  raw transcript

│

▼

Keyword guard  →  off-topic? return 400

│

▼

mBERT classifier  →  intent + confidence

│

▼

Confidence gate  →  < 75%? return UNCERTAIN

│

▼

execute_action()  →  serial: RELAY:1:ON\n

│

▼

Arduino Uno  →  relay toggle  →  bulb ON/OFF

│

▼

SQLite log  +  JSON response  →  dashboard update

# 3. System Requirements

## 3.1  Hardware

| Component | Specification |
| --- | --- |
| PC / Laptop | Windows 10/11, 64-bit, minimum 8 GB RAM (16 GB recommended for Whisper) |
| Arduino Uno | Any revision — Rev3 is most common |
| Relay module | 1-channel or 2-channel relay module (5V coil) |
| DC bulb | Any DC bulb matching your relay's rated voltage/current |
| USB cable | USB Type-A to Type-B (standard Arduino cable) |
| Jumper wires | Male-to-male or male-to-female, for relay wiring |
| Microphone | Laptop built-in mic works; headset or USB mic gives better accuracy |

## 3.2  Software

| Software | Notes |
| --- | --- |
| Anaconda | Python environment manager — miniconda also works |
| Python 3.11 | Specified when creating the conda environment |
| Arduino IDE | Version 2.x — for uploading the sketch to the board |
| Google Chrome / Edge | For the dashboard voice recording (Firefox may block mic API) |
| ffmpeg | Included in the nlp/ folder — required by Whisper for audio conversion |

# 4. Installation

## 4.1  Install Anaconda

Download Anaconda from https://www.anaconda.com/download  and run the installer. Accept all defaults. Once installed, open Anaconda Prompt (not regular PowerShell) for all commands in this guide.

## 4.2  Create the Python Environment

Run these commands once in Anaconda Prompt:

ANACONDA PROMPT

conda create -n smarthome python=3.11 -y

conda activate smarthome

You should see (smarthome) at the start of your prompt after activation.

## 4.3  Install Python Dependencies

With the smarthome environment active, install all required packages:

ANACONDA PROMPT

pip install torch --index-url https://download.pytorch.org/whl/cpu

pip install transformers==4.40.0

pip install flask openai-whisper pyserial sounddevice numpy requests scikit-learn gpiozero

conda install -c conda-forge ffmpeg -y

⚠️ Important — ffmpeg

The nlp/ folder already contains ffmpeg-8.1.1-essentials_build/ which Whisper uses for audio conversion. If the conda ffmpeg install above fails or crashes, the bundled one will be used automatically. Do not delete the ffmpeg folder.

## 4.4  Install Arduino IDE

Download Arduino IDE 2.x from https://www.arduino.cc/en/software and install it. No additional libraries are needed — the relay sketch uses only built-in Arduino functions.

# 5. Folder Contents

After extracting the nlp.zip archive, your folder should contain exactly these files:

| File / Folder | Purpose |
| --- | --- |
| app.py | Flask API server — the main backend. Run this first. |
| voice_client.py | Terminal voice client using Whisper + microphone |
| phonetic_corrector.py | STT correction layer — auto-imported by voice_client.py |
| generate_metrics.py | Prints evaluation metrics from the database |
| smart_home.db | SQLite database — logs all commands and energy sessions |
| saved_relay_model/ | Fine-tuned mBERT model weights, tokenizer, and config |
| templates/index.html | Web dashboard UI — served automatically by Flask |
| ffmpeg-8.1.1-essentials_build/ | ffmpeg binaries required by Whisper for audio conversion |

# 6. Configuring the Server

Open app.py in any text editor. Near the top, find the CFG dictionary. The key settings to review before first run are:

app.py — CFG section

CFG = {

"arduino_port"    : None,     # Set to "COM3" (or your port) once Arduino is connected

"arduino_baud"    : 9600,     # Must match the baud rate in the Arduino sketch

"arduino_channel" : 1,        # Relay channel number

"arduino_timeout" : 2,        # Seconds to wait for ACK from Arduino

"default_wattage" : 5.0,      # Watts — your bulb's rated wattage

"whisper_model"   : "small",  # STT model: tiny / base / small / medium

"min_confidence"  : 0.75,     # Intent confidence threshold (0.0 – 1.0)

}

💡 Tip — Arduino Port

If arduino_port is set to None, the server will auto-detect the Arduino by scanning all COM ports for known USB descriptors (CH340, FTDI, Arduino Uno). If auto-detect fails, set the port explicitly. To find your COM port: open Device Manager → Ports (COM & LPT) after plugging in the Arduino.

# 7. Running the System

## 7.1  Start the Flask Server

Open Anaconda Prompt, navigate to the nlp/ folder, and run:

ANACONDA PROMPT — TERMINAL 1

conda activate smarthome

cd C:\path\to\nlp

python app.py

A successful startup prints:

EXPECTED OUTPUT

✅  Database ready: ./smart_home.db

✅  Arduino connected on COM3 @ 9600 baud   (or: ⚠️  Running in SOFTWARE SIMULATION mode)

✅  mBERT loaded on CPU

✅  Whisper STT ready.

════════════════════════════════════════

Smart Home API Gateway  –  Arduino Mode

http://127.0.0.1:5000

════════════════════════════════════════

## 7.2  Open the Dashboard

Once the server is running, open Google Chrome or Microsoft Edge and navigate to:

BROWSER ADDRESS BAR

http://127.0.0.1:5000

The SHEMS Control Hub dashboard will load. It automatically polls the server every 2 seconds and updates all panels in real time.

## 7.3  Send Commands

Via the dashboard (Text):

- Type any command in the text box — e.g. Turn on the light or Washa taa

- Press Enter or click Send

- The response panel shows the detected intent, confidence, and bilingual feedback

Via the dashboard (Voice):

- Click Record Command — the button turns red

- Speak your command clearly

- Click again to stop — or it auto-stops after 6 seconds

- Whisper transcribes locally, mBERT classifies, Arduino actuates

Via the terminal voice client (optional):

ANACONDA PROMPT — TERMINAL 2

conda activate smarthome

python voice_client.py

ℹ️ Software Simulation Mode

If the Arduino is not connected, the server runs in simulation mode. All NLP classification, logging, energy tracking, and dashboard updates work normally. The relay state is tracked in software instead of sent over serial. This is useful for testing without hardware.

# 8. API Reference

All endpoints are available at http://127.0.0.1:5000. They can be called from any HTTP client.

| Endpoint | Description |
| --- | --- |
| POST /api/command | Send a text command. Body: {"text": "Turn on the light"} |
| POST /api/voice | Send a voice recording. Body: multipart/form-data with 'audio' field |
| GET /api/status | Server status, relay state, uptime, Arduino connection info |
| GET /api/logs | Last 10 command log entries from the database |
| GET /api/energy | Energy sessions, live duration, total Wh consumed |
| POST /api/energy/wattage | Update bulb wattage. Body: {"wattage": 9.0} |
| POST /api/energy/cost | Update KSh/kWh tariff. Body: {"cost_per_kwh": 23.0} |
| GET /api/tips | Dynamic energy saving tips based on usage patterns |
| GET /api/arduino/ports | List all available serial ports on the PC |
| POST /api/arduino/config | Update Arduino port/baud/channel at runtime without restart |
| GET / | Web dashboard UI |

# 9. Arduino Uno Integration

## 9.1  Wiring the Relay Module

Connect the relay module to the Arduino Uno using jumper wires:

| Relay Pin | Connect to |
| --- | --- |
| Relay VCC | Arduino 5V |
| Relay GND | Arduino GND |
| Relay IN1 | Arduino Digital Pin 7 (configurable in sketch) |
| Relay COM | One terminal of the DC bulb circuit |
| Relay NO | Power supply positive terminal (Normally Open — circuit open when relay is off) |

⚠️ Safety — Relay Wiring

Always check the relay module's rated voltage and current before wiring. Most 5V relay modules support up to 10A / 250VAC or 30VDC. Ensure your DC bulb and power supply are within these limits. For a DC circuit: connect the bulb in series between the relay COM and NO terminals and the power supply.

## 9.2  Arduino Sketch

The person responsible for the Arduino sketch must implement this serial communication protocol to work with the Flask server:

ARDUINO SKETCH (reference — for the hardware developer)

// Arduino Uno — SHEMS Relay Controller

// Upload via Arduino IDE before connecting to Flask

const int RELAY_PIN = 7;        // Change to match your wiring

const int BAUD_RATE = 9600;     // Must match arduino_baud in CFG

void setup() {

Serial.begin(BAUD_RATE);

pinMode(RELAY_PIN, OUTPUT);

digitalWrite(RELAY_PIN, LOW);  // Start with relay OFF

Serial.println("SHEMS_READY");

}

void loop() {

if (Serial.available()) {

String cmd = Serial.readStringUntil('\n');

cmd.trim();

// Expected format: RELAY:<channel>:<ON|OFF>

if (cmd.startsWith("RELAY:")) {

if (cmd.endsWith(":ON")) {

digitalWrite(RELAY_PIN, HIGH);

Serial.println("ACK:ON");

} else if (cmd.endsWith(":OFF")) {

digitalWrite(RELAY_PIN, LOW);

Serial.println("ACK:OFF");

}

}

}

}

📋 Serial Protocol Summary

Flask sends: RELAY:1:ON\n or RELAY:1:OFF\n Arduino replies: ACK:ON\n or ACK:OFF\n Baud rate must match on both sides (default: 9600) Arduino must reply within arduino_timeout seconds (default: 2s) or Flask logs a warning

## 9.3  Connecting the Arduino

- Upload the sketch using Arduino IDE (Sketch → Upload)

- Unplug the Arduino from the PC, then reconnect it — this resets the board

- Note the COM port from Device Manager (e.g. COM3, COM4)

- Set the port in app.py:

app.py — CFG

"arduino_port" : "COM3",    # replace with your actual port

- Start the Flask server — it will connect automatically on startup

- Or update the port at runtime without restarting — open a new terminal and run:

ANACONDA PROMPT — runtime reconfiguration

curl -X POST http://127.0.0.1:5000/api/arduino/config ^

-H "Content-Type: application/json" ^

-d "{\"port\": \"COM3\", \"channel\": 1}"

# 10. Dashboard Guide

The dashboard at http://127.0.0.1:5000 has five sections:

| Panel | Description |
| --- | --- |
| GPIO17 Relay — DC Bulb | Visual bulb indicator. Glows gold when ON, dark when OFF. Updates every 2 seconds. |
| Commands Logged | Total transactions, valid vs filtered breakdown, and server uptime. |
| Energy Snapshot | Live session duration and mWh, plus cumulative totals. Ticks every 2 seconds while bulb is ON. |
| Bulb Configuration | Set bulb wattage (W) and Kenya Power tariff (KSh/kWh) — used for cost estimates. |
| Command Interface | Text input and voice recording button. Responses show intent, confidence, and bilingual feedback. |
| Energy Insights | Dynamic tips generated from your actual usage patterns. Refreshes every 10 seconds. |
| Audit Trail table | Last 10 commands with intent, confidence bar, language, and relay state. |
| Energy Sessions table | Per-session ON/OFF timestamps, duration in seconds, and mWh consumed. |

# 11. Troubleshooting

| Error / Symptom | Fix |
| --- | --- |
| ModuleNotFoundError: No module named 'torch' | The smarthome environment is not active. Run: conda activate smarthome |
| Arduino not found — running in simulation mode | Check USB cable, open Device Manager to confirm COM port, then set arduino_port in CFG or call /api/arduino/config |
| Voice recording not working in browser | Use Chrome or Edge. Firefox blocks MediaRecorder on localhost in some versions. Ensure microphone permission is granted. |
| Whisper error: Failed to load audio | ffmpeg is not found. Ensure ffmpeg-8.1.1-essentials_build/bin/ exists in the nlp/ folder and app.py has the correct FFMPEG path. |
| Intent always returns UNKNOWN/NOISE | The command does not contain any domain keywords. Add the word to domain_keywords in CFG, or rephrase the command. |
| Low confidence / UNCERTAIN | Speak closer to the microphone, reduce background noise, or use the text input instead. |
| OMP: Error #15 on startup | Add os.environ["KMP_DUPLICATE_LIB_OK"] = "TRUE" at the very top of app.py (already included by default). |
| Port already in use: 5000 | Another process is using port 5000. Change "port": 5000 in CFG to 5001 or kill the other process. |

# 12. Generating Evaluation Metrics

After using the system, run the metrics script to get a report suitable for academic documentation:

ANACONDA PROMPT

conda activate smarthome

python generate_metrics.py

The report includes: total transactions, noise rejection rate, intent distribution, language distribution, average model confidence, relay actuation summary, and estimated energy cost.

# 13. Quick Reference

## Start the system (every time)

ANACONDA PROMPT

conda activate smarthome

cd C:\path\to\nlp

python app.py

# Then open http://127.0.0.1:5000 in Chrome

## Example commands

| Command | Intent |
| --- | --- |
| Turn on the light | TURN_ON |
| Washa taa | TURN_ON |
| Switch off the bulb | TURN_OFF |
| Zima taa | TURN_OFF |
| Is the light on? | GET_STATUS |
| Hali ya taa ni nini? | GET_STATUS |
| How can I save energy? | GET_ADVICE |
| Nipe ushauri wa nishati | GET_ADVICE |

## Key file locations

| Item | Path |
| --- | --- |
| Flask server | nlp/app.py |
| Dashboard UI | nlp/templates/index.html |
| Database | nlp/smart_home.db |
| mBERT model | nlp/saved_relay_model/ |
| ffmpeg binaries | nlp/ffmpeg-8.1.1-essentials_build/bin/ |
| Metrics script | nlp/generate_metrics.py |

SHEMS Control Hub  ·  JKUAT Final Year Project  ·  NLP Smart Home Energy Management